ALUMINUM WIRING PROCEDURES

Listed below are the procedures mandated by company policy at all times when working with aluminum wiring:

  - The circuit breaker boxes must be inspected to insure that the lug nuts are tight and to apply the product, “Noalox”, an antioxidant joint compound.

  - Purple wire nuts (Ideal #30-265, #30-365 – twisted al/cu) that are rated for use with aluminum to copper pigtail connections.  These wire nuts are to be used in the installation of all the light fixtures, ceiling fans, and other hard wired appliances.

  - CO/ALR rated switches and plugs are the only switches and plugs allowed on the property.  When removing a copper rated only, i.e. not CO/ALR plug or switch, please take to the office after proper replacement for the property manager or maintenance supervisor to investigate.

The above procedures are to be followed in all vacant units at time of turn, periodically in occupied units, and at time of requested repair service.

Please sign and date below to indicate that you understand all above listed procedures.

____________________________			______________________________

Employee 						Date

___________________________			______________________________

Property Manager					Date